LAB Assignment No 6

ABDULLAH SOHAIL

70147165

Interfacing LED & Potentiometer with ESP32 and Visualizing Data on Node-RED Dashboard

LAB Description:

This lab demonstrates how IoT devices communicate with cloud-based dashboards using MQTT, enabling real-time monitoring and control.

Learning Objectives:

By the end of this lab, students will:

- Understand IoT communication using MQTT

- Interface sensors (potentiometer) with ESP32

- Control actuators (LED) remotely

- Build a web dashboard using Node-RED

- Simulate IoT systems using Wokwi

Required Tools

🔹 Hardware (Simulated)

- ESP32

- LED

- Potentiometer

🔹 Software

- Arduino IDE

- Node-RED

- MQTT Broker (e.g., HiveMQ or Mosquitto)

System Overview

Potentiometer → ESP32 → MQTT → Node-RED Dashboard (Display)

Node-RED Button → MQTT → ESP32 → LED Control

Potentiometer:

- VCC → 3.3V

- GND → GND

- OUT → GPIO 36

LED:

- Positive → GPIO 26

- Negative → GND

ESP32 Tasks

Students must:

- Connect ESP32 to WiFi

- Read potentiometer value

- Publish data to MQTT topic:

pubtopic

- Subscribe to MQTT topic:

LED

- Turn LED ON/OFF based on received message

ESP32 code

#include <WiFi.h> #include <PubSubClient.h> // ================= WiFi ================= const char* ssid = "Wokwi-GUEST"; const char* password = ""; // ================= MQTT ================= const char* mqttServer = "broker.hivemq.com"; const int mqttPort = 1883; // ================= Pins ================= #define ADCpin 36 #define LED 26 int ValueToSend; // ================= Objects ================= WiFiClient espClient; PubSubClient client(espClient); // ================= CALLBACK ================= void callback(char* topic, byte* payload, unsigned int length) { Serial.println(); Serial.print("Message received on topic: "); Serial.println(topic); String msg = ""; for (int i = 0; i < length; i++) { msg += (char)payload[i]; } Serial.print("Message: "); Serial.println(msg); if (String(topic) == "LED") { if (msg == "ON") { digitalWrite(LED, HIGH); Serial.println("LED ON"); } else if (msg == "OFF") { digitalWrite(LED, LOW); Serial.println("LED OFF"); } } } // ================= WIFI CONNECT ================= void connectWiFi() { Serial.print("Connecting to WiFi"); WiFi.begin(ssid, password); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } Serial.println("\nWiFi Connected!"); Serial.print("IP Address: "); Serial.println(WiFi.localIP()); } // ================= MQTT RECONNECT ================= void reconnectMQTT() { while (!client.connected()) { Serial.println("Connecting to MQTT..."); String clientId = "ESP32Client-"; clientId += String(random(1000)); // PUBLIC BROKER → NO USERNAME/PASSWORD if (client.connect(clientId.c_str())) { Serial.println("MQTT Connected!"); client.subscribe("LED"); // Subscribe topic } else { Serial.print("Failed, state = "); Serial.println(client.state()); delay(2000); } } } // ================= SETUP ================= void setup() { Serial.begin(115200); pinMode(LED, OUTPUT); connectWiFi(); client.setServer(mqttServer, mqttPort); client.setCallback(callback); } // ================= LOOP ================= void loop() { if (!client.connected()) { reconnectMQTT(); } client.loop(); // Read ADC value int ADC = analogRead(ADCpin); ValueToSend = map(ADC, 0, 4095, 0, 100); Serial.print("Pot Value (%): "); Serial.println(ValueToSend); // Convert to string char payload[10]; snprintf(payload, sizeof(payload), "%d", ValueToSend); // Publish to MQTT client.publish("pubtopic", payload); delay(2000); }

Viva Questions:

- What is MQTT and why is it used in IoT?

- Difference between publish and subscribe?

- Why do topics need to match exactly?

- What happens if broker is down?

- What is the role of Node-RED?