LAB Assignment No 7

Interfacing DHT22 Sensor with ESP32 and Visualizing Temperature & Humidity on Node-RED Dashboard

Lab Description:

This lab demonstrates real-time environmental monitoring using IoT, where sensor data is transmitted via MQTT and visualized using a web dashboard.

Learning Objectives:

By the end of this lab, students will:

- Interface digital sensors (DHT22) with ESP32

- Publish real-time data using MQTT

- Visualize IoT data using Node-RED

- Build dashboards with charts and gauges

- Simulate IoT systems using Wokwi

ESP32 Code:

#include <WiFi.h> #include <PubSubClient.h> #include "DHTesp.h" const int DHT_PIN = 9; DHTesp dhtSensor; const char* ssid = "Wokwi-GUEST"; const char* password = ""; const char* mqtt_server = "broker.hivemq.com"; WiFiClient espClient; PubSubClient client(espClient); unsigned long lastMsg = 0; #define MSG_BUFFER_SIZE (50) float temp = 0; float hum = 0; int value = 0; void setup_wifi() { delay(10); // We start by connecting to a WiFi network Serial.println(); Serial.print("Connecting to "); Serial.println(ssid); WiFi.mode(WIFI_STA); WiFi.begin(ssid, password); while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); } randomSeed(micros()); Serial.println(""); Serial.println("WiFi connected"); Serial.println("IP address: "); Serial.println(WiFi.localIP()); } void callback(char* topic, byte* payload, unsigned int length) { Serial.print("Message arrived ["); Serial.print(topic); Serial.print("] "); for (int i = 0; i < length; i++) { Serial.print((char)payload[i]); } Serial.println(); // Switch on the LED if an 1 was received as first character if ((char)payload[0] == '1') { digitalWrite(2, LOW); // Turn the LED on (Note that LOW is the voltage level // but actually the LED is on; this is because // it is active low on the ESP-01) } else { digitalWrite(2, HIGH); // Turn the LED off by making the voltage HIGH } } void reconnect() { // Loop until we're reconnected while (!client.connected()) { Serial.print("Attempting MQTT connection..."); // Create a random client ID String clientId = "ESP8266Client-"; clientId += String(random(0xffff), HEX); // Attempt to connect if (client.connect(clientId.c_str())) { Serial.println("Connected"); // Once connected, publish an announcement... client.publish("IoT/mqtt", "IoT"); // ... and resubscribe client.subscribe("IoT/mqtt"); } else { Serial.print("failed, rc="); Serial.print(client.state()); Serial.println(" try again in 5 seconds"); // Wait 5 seconds before retrying delay(5000); } } } void setup() { pinMode(2, OUTPUT); // Initialize the BUILTIN_LED pin as an output Serial.begin(115200); setup_wifi(); client.setServer(mqtt_server, 1883); client.setCallback(callback); dhtSensor.setup(DHT_PIN, DHTesp::DHT22); } void loop() { if (!client.connected()) { reconnect(); } client.loop(); unsigned long now = millis(); if (now - lastMsg > 2000) { lastMsg = now; TempAndHumidity data = dhtSensor.getTempAndHumidity(); String temp = String(data.temperature, 2); Serial.print("Temperature: "); Serial.println(temp); client.publish("IoT/temperature", temp.c_str()); String hum = String(data.humidity, 1); Serial.print("Humidity: "); Serial.println(hum); client.publish("IoT/humidity", hum.c_str()); } }